Running the DIAG508 Customer Diagnostics

There are four items you should check on your network

before running the DIAG508.COM diagnostics routines:

1)  The interrupt  switches  on  all ARCnet Lan interface cards

must be set to the  parameters  required by the version  of

network software that you are running. For NetWare/86 1.01,

all workstation  interrupts   must be  disabled.  All later

releases,  as  well as  Advanced NetWare/286  requires  ALL

stations to have IRQ 2 on.

2)  Regardless  of  which  network  software  package  you  are

using,  each LAN interface card on the network  must have a

unique network address.  For  all  versions of  NetWare 2.0

and greater, set the fileserver network address to a  value

less than 0F hex, and all workstations to a value of 10 hex

or greater.

3)  The memory address  and I/O  addressses  must  be  properly

configured for the shell(s)  which  you  are  running.  For

NetWare, the default  parameters  are memory address D000:0

and  I/O  address 2E0.  ANet286 2.0  and greater allows you

to  custom  configure  the  fileserver's   memory  and  I/O

addresses  when   running   the  Genos  utility.  The  card

configuration(s)  in  the  fileserver   must   match  those

selected during operating system creation.

4)  The cable installation for your network must conform to the

standards which are  laid  out in the  PDC508A/P  Guide  to

Installation and Operation.

After checking all of the above, and your network is  still

not functioning properly, one of the ARCnet products may be

at  fault.  If  only  one  station  on  the  network is not

functioning, you  should test  the 508Plus card  from  that

station  first.  If  your entire network or portion thereof

is down, you should try to isolate  the offending  hardware

by disconnecting cables one at a time from the hub(s) until

the remaining system(s) resume normal operation.  Once  you

have narrowed down the source of the problem to one area of

the network, go through the remaining hardware as  outlined

to track down the culprit.

NOTE !!

~~~~~~~

If you disconnect a workstation  that is  attached to a

passive hub, you  must  install  a  terminator  on  the

unused port of the passive hub.

Once you have isolated a Pure Data ARCnet product  that you

suspect  of  being  faulty,  prepare  to  run  the  DIAG508

Diagnostics.  There are two options to choose from when you

run  the  DIAG508  Diagnostics  routine.  You  can  test  a

508Plus card  by itself  in  a PC, or you can  install  the

suspect card in a PC  with  another  508Plus card  that you

know is working properly.  The two card  testing  procedure

is more comprehensive, because the cards can "talk" to each

other.  This will assist in pinpointing the characteristics

of the failure and thus its probable cause.   By installing

a hub between  two  known  functional cards and running the

two  card test, all ports of the hub can be tested to check

for proper operation.   Decide  which testing procedure you

wish to use and follow the  appropriate  setup instructions

below.

IMPORTANT:  Ensure that the board(s) under test is/are NOT connected to

the  network  while  you  are running the Diagnostics.  The

test program will crash the  network  if  you  run  it on a

station that is linked to the rest of the system.

Testing Preparation

Install one or two known good  PDI508Plus  card(s) in a  PC

and  run  the  diagnostics  as outlined  below  (either the

single or two card test) to benchmark the system.   In this

way,  when  one  of  the  functional  cards  is removed and

replaced  with  an  unqualified  board, the origin  of  the

failure is undoubtedly the suspect card.

Setup For Single Board Test

1)  We suggest that you test the suspect board in the benchmarked

PC (procedure described above).  You should remove the PC's

cover for the duration of the test.

2)  Boot up the  PC  under  DOS; do not  run the shell.  Ensure

that the PC is  disconnected from the  rest of the network.

Insert  the  PDI508Plus  Diagnostics  diskette in drive B:,

and at the A> prompt enter:

B:DIAG508 <return>

3)  The Diagnostics routines will guide you through the test.

Setup For Two-Board Test

1)  Power-off the PC with the  suspect  card  and   remove  the

PDI508Plus  card.  Install  the  suspect  PDI508Plus in the

previously described benchmarked PC, along with a card that

you know is working correctly.  Once the cover of the PC is

removed, leave it off for the duration of the testing.

2)  Connect the two cards with  a length of RG62/U coax cable.

3)  Boot  up  the  PC  under  DOS.  Ensure  that  the   PC   is

disconnected from the rest of the network and  do  not  run

the shell. Insert the 508Plus Diagnostics diskette in drive

B, and at the A> prompt enter:

B:DIAG508 <return>

4)  The Diagnostics routines will guide you through the test.

5)  If the tests pass the suspect  card, but  the  card  itself

fails intermittently in  use,  reinstall the PC's cover and

set the test to run indefinitely.  While testing the card's

abilities, the extra thermal stressing will push it to  the

limits.

Setup for Active Hub Testing

1)  Follow the setup instructions for the  two  card  test, but

use two known functional cards and cables so that the only

variable in the test is the hub.

2)  Apply power to the active hub.

3)  Connect one of the cards to port one of the active hub, and

the other card to port number two.

4)  Boot  up  the  PC  under  DOS.  Ensure  that  the   PC   is

disconnected from the rest of the network and  do  not  run

the shell. Insert the 508Plus Diagnostics diskette in drive

B, and at the A> prompt enter:

B:DIAG508 <return>

5)  Follow the menues through to the two card test.  Allow  the

communications test to run  for  at  least  10  seconds  to

verify  that  the  two  channels  are  operational.  Strike

return, or just disconnect the cable from  the  port  under

test.  Either action will cause the test to terminate.

6)  Move the cable from the second port to the third to prepare

to test the first and third ports for proper operation.

7)  Select the retry option to test this pair of ports.

8)  Continue stepping through the nodes until you have verified

node one with respect to the other seven.

Pure Data Technical Support : CANADA (416) 731 9884

USA    (800) 661 8210

UK     011-44-71-924-3505

Pure Data BBS (2400,n,8,1)  : CANADA (416) 492 5980

USA    (214) 242-3225

UK     011-44-71-738-2910