INSTALLATION PROCEDURE

------------------------------------------------------------------------

Using the Pure Data High Performance Drivers

This addendum is intended to provide you with a few installation

hints and guidelines particular to the Pure Data High Performance

Drivers.

These Drivers, only available for version 2.1x of Novell, offer more

configuration options when generating the Operating system and the

workstation shell. They also allow users to install up to four ARCnet

cards in one Fileserver. In order for the AT/XT Drivers to function in

High Performance mode, a PDI516 has to be installed in the Fileserver.

For the PS/2 drivers, as long as the PDuC508 card is installed in the

server, the drivers will be in High Performance mode.

When using a PDI516Plus in the Fileserver, the card number (jumper E2)

will have to be set according to the LAN selection made in Netgen.

Unpredictable occurences may be encountered if this card numbering

scheme is not followed properly.

E2 set for Card 1 (or A)  -  Netgen   LAN A

Card 2 (or B)  -  Netgen   LAN B

Card 3 (or C)  -  Netgen   LAN C

Card 4 (or D)  -  Netgen   LAN D

IMPORTANT NOTICE

----------------

We recommend setting the High Performance Drivers to a maximum packet

size of 512 Bytes if an 8 Bit card is installed in the Fileserver. Since

the High Performance Drivers only operate in High Performance mode if a

PDI516Plus is detected in the Fileserver, using them with an 8Bit card

and with any packet size other than 512, will not give any performance

improvement.  If in fact the 512 Bytes size is used on the Fileserver,

the shell should be set to the same size.

NOTE: Whatever size the Fileserver Drivers are set to operate at, always

keep the shell at a smaller or equal packet size. To set the packet size

of the IPX.COM or the NET$OS.EXE, use the utility SETSIZE.EXE. The

detailed explanation on how to patch a file can be found in SETSIZE.DOC

(the two SETSIZE files can be found on most Support Diskettes). If the

workstations are not using any of the Pure Data Adapters (PDI507,

PDI508Plus, PDI516Plus and PDuC508), the shell drivers will function in

standard mode as long as a compatible card is used.

These Drivers are only for Netware 2.1 and higher.

------------------------------------------------------------------------

Installation hints for Netware 2.1x:

-----------------------------------

Creating the NET$OS.EXE file from floppy:

- Copy the Drivers found in the NETWARE/2.1 subdirectory of

the Support Diskette, in the root directory of a disk

labelled LAN_DRV_508. (NOTE: You may use any of the Pure

Data Support Diskettes labelled LAN_DRV_508)

- Run  NETGEN

- Select option "Select LAN drivers", followed by option

"Load and Select item".  When prompted insert the diskette

labelled LAN_DRV_508 (on which the new drivers were

copied). Then select the PURE DATA HIGH PERFORMANCE

option.

- At this point continue with selection as you would for

a normal NETGEN.

Creating the NET$OS.EXE file from a hard drive or Network Drive::

- Create a subdirectory called LAN_DRV_.508

- Copy the PDL drivers from the \NETWARE\2.1 subdirectory

into the LAN_DRV_.508 subdirectory.

- Run  NETGEN

- Select option "Select LAN drivers", followed by option

"Load and Select item". This will load the new drivers

found in subdirectory LAN_DRV_.508. Then select the PURE

DATA HIGH PERFORMANCE option.

- At this point, continue with normal selection for

NETGEN.

NOTE:  A maximum of four cards will be allowed in the server.

Creating the IPX.COM file:

- Run "SHGEN -N" and select "Load and Select Item". (If

you are executing the shell generation from floppy, you

will have to insert the LAN_DRV_508 diskette when

prompted).  After the new shell drivers have been loaded

select the option PURE DATA HIGH PERFORMANCE.

NOTE:  The shell generated with Pure Data's PS2 Drivers, and the AT/XT

Drivers (only when used on a PDI516), will dynamically and automatically

reconfigure themselves to match the settings of any PDuC508 or

PDI516Plus cards found in the computer (i.e. only one shell generation

is necessary). If the shell generated for an AT/XT type machine is used

on standard 8 bit Pure Data cards, you may use Pure Data's patching

utility (SHLGGEN) to patch your Pure Data shell for any settings

required.