PURE DATA PACKET SIZE CONFIGURATION UTILITY V1.2  (Mar 21,1990)

----------------------------------------------------------------

1.  Description:

-----------

The SETSIZE utility is used to change the maximum packet size of a Pure

Data driver used with Novell Netware V2.1X.  Both the NET$OS and IPX files

can be modified with this utility.  Each valid driver found in an

executable file will be modified.  (NOTE:  remember to flag the file

Read/Write before patching).

The OS driver with the largest packet size will determine how much

of the OS memory will be dedicated to driver usage.  If the OS

configuration results in less than 3 process buffers, the maximum packet

size should be reduced (with too few process buffers the network

performance will be very choppy).  The Pure Data drivers are shipped with

2K packets selected, leaving 3 free process buffers on a standard file

server.

NOTE:  The OS drivers must not have a smaller maximum packet size than the

shell driver.

2.  Minimum Driver Revisions:

------------------------

The drivers must be designed in  a  manner  which  permits  them  to  be

patched.  Only  the  following Pure Data drivers and any later versions will

work with the SETSIZE.EXE utility. If the  driver  is  not  compatible,  the

driver  will  not  be  modified  and the warning message "No change, invalid

driver" will be displayed. The type and minimum driver revisions  which  can

be used are:

PURE DATA ARCNET Enhanced Driver V3.0                   (or greater)

PURE DATA ARCNET Enhanced Driver V3.0 License #******** (or greater)

PURE DATA MICRO CHANNEL ARCNET Enhanced Driver V3.0     (or greater)

PURE DATA TOKEN RING (V1.1)                             (or greater)

PURE DATA MICRO CHANNEL TOKEN RING (V1.0)               (or greater)

3.  Usage:

-----

The SETSIZE function operates on the linked executable file: The OS

file (NET$OS.EXE) & the Shell file (IPX.COM). If the OS file has more than 1

driver present, all the appropriate ones will be patched according to the /p

option.

When a driver is encountered its description and packet size will be

displayed, as well as a status message stating the changes applied to the

driver.  To display the packet size of the Shell or OS without altering it,

just run "SETSIZE /f:Filename.xxx" without the /p option.

NOTE: We recommend running SETSIZE.EXE on a copy of NET$OS.EXE rather

than the original file.

The SETSIZE.EXE utility is used as follows:

[drive path:] setsize  [parameters]

Parameters     Options                                     Default

/f:xxxxxx.xxx  Path and Filename of object file to use     none

/p:xxxx        Packet size for IPX (512/1024/2048/4096)    none

?             Display help information

Note: Do not enclose optional parameters within the [] symbols.

EXAMPLE: Patching NET$OS.EXE to 1024 byte packets

Type the following:

flag        net$os.exe   rw

setsize  /f:net$os.exe   /p:1024

flag        net$os.exe   ro

Response of setsize.exe:

Driver 1  Description: PURE DATA ARCNET Enhanced Driver V3.0

Driver 1  Packet Size: 1024 (Updated)

Notes:

The modified NET$OS.EXE will take effect the next

time the system is turned on.

4.  Status Messages:

---------------

The report displayed for each driver encountered will be accompanied  by

a status message. The following messages may occur:

(Updated)

(Write corrupted, file may be read only)

(No change requested)

(No change, invalid revision)

(No change, invalid driver)

5.  Detailed Background:

-------------------

Netware uses an area in memory for data storage as well as for its

File Server Process Buffers.  These buffers are used by Netware to pass

data between the LAN and the file system.  With each available buffer, a

separate process can conduct a transaction between the file system and the

LAN.  This has the effect that multiple requests can be processed in

parallel.  When all buffers are allocated, no further processing is

possible and requests are queued until a file process buffer becomes

available.

Normally, when a driver declares the packet sizes it supports, Netware

divides a portion of that memory into contiguous chunks large enough to

support the largest driver in the fileserver.  These chunks are the File

Process Buffers.  With 4K packets, there is usually only room for 2

buffers, hence there seems to be a maximum of 8K of memory set aside for

the File Process Buffers.

Since reducing the number of communication buffers to a minimum of 3

might not suit everybody's purposes (specially if more than 2 LAN

Cards are used in the fileserver), SETSIZE may be used to reduce the size

of the packets, therefore increasing the number of Buffers. Remember that

reducing the size of the packets will at the same time reduce the Network

performance.  The reduction in network performance will be more pronounced

when using a workstation with a low cpu speed.

As a rough rule of thumb, you should ensure that you have at least one

file service process available for each NIC in the fileserver.  If you have

a VAP that makes access to the file system, then you could allow an

additional file service process for it as well.

NOTE: The Fileserver's maximum packet size should not be less than the

shell's.